PacketForge V1.0

Phase 6 - M7 Failure & Lifecycle Validation

GAP-035 - Peer Disconnect During Workflow

Closure and Test Execution Record

| Property | Value |
| --- | --- |
| Document ID | PF-V1-P6-M7-G035-CR-001 |
| Phase | Phase 6 - Application & Client-Server Integration |
| Milestone | M7 - Failure & Lifecycle Validation |
| Gap | GAP-035 - Peer Disconnect During Workflow |
| Status | CLOSED / PASS |
| Test Target | PacketForge V1.0 server application |
| Test Platform | Linux / WSL |
| Server Endpoint | 127.0.0.1:9090 |

# 1. Objective

GAP-035 validates the behavior of the real PacketForge V1.0 server when the connected peer disconnects abruptly during an active request/response transaction. The disconnect must occur after the server has received and dispatched a valid request, but before the server completes response transmission.

The objective is not to validate the low-level send API in isolation. It is to validate the complete application path: request reception, dispatch, deterministic synchronization, peer reset, response transmission attempt, failure detection, error propagation, cleanup, and process termination.

# 2. GAP-035 Definition

GAP-035 addresses the following lifecycle condition:

Client connects
    |
    | Valid HelloRequest
    v
Server receives request
    |
    v
Server dispatches request
    |
    v
SERVER PAUSED immediately before packetIO.send(response)
    |
    | Client performs TCP RST
    v
Server resumes
    |
    v
packetIO.send(response)
    |
    v
Send failure handling
    |
    v
Centralized cleanup
    |
    v
Server exits with failure status

The gap is intentionally scoped to the existing synchronous, blocking V1.0 architecture. No multiplexing, asynchronous I/O, non-blocking sockets, or architecture changes are introduced by the test.

# 3. Relationship to Previously Closed Gaps

| Gap | Coverage | Why GAP-035 is different |
| --- | --- | --- |
| GAP-026 | Disconnect / lifecycle after peer closure | Focuses on the lifecycle once peer closure has been observed and on post-close behavior. |
| GAP-033 | Transport and PacketIO send failure | Validates low-level send failure and propagation in isolation. |
| GAP-034 | Malformed packet handling | Validates protocol/decode rejection of malformed input. |
| GAP-035 | Peer disconnect during active workflow | Validates the real server application path when the peer disappears immediately before response transmission. |

# 4. TCP RST Concept Used by the Test

Before describing the test procedure, it is important to define RST because the GAP-035 injection depends on an abortive TCP close.

## 4.1 What is RST?

RST means TCP Reset. It is the TCP reset flag used to abort a connection rather than performing the normal FIN/ACK connection shutdown sequence.

## 4.2 Normal TCP close versus RST

A normal TCP close is an orderly shutdown:

Client                         Server
  |                              |
  |----------- FIN ------------->|
  |<---------- ACK --------------|
  |                              |
  |<----------- FIN -------------|
  |----------- ACK ------------->|
  |                              |
  connection closes normally

The GAP-035 test instead uses an abortive close:

Client                         Server
  |                              |
  |----------- RST ------------->|
  X                              |
  connection is aborted          |

RST is used here because the test needs to model an abrupt peer disappearance while the server is still in the middle of a request/response transaction.

## 4.3 How the test client generates RST

The dedicated GAP-035 client enables SO_LINGER with l_onoff = 1 and l_linger = 0, then disconnects the PacketForge connection. This produces an abortive TCP close rather than the normal graceful FIN-based close.

struct linger lingerOption
{
    1,
    0
};

setsockopt(
    socket,
    SOL_SOCKET,
    SO_LINGER,
    &lingerOption,
    sizeof(lingerOption)
);

transport.connection().disconnect();

# 5. Server Termination Behavior

## 5.1 Normal successful termination

In the normal successful workflow, the server receives and processes a valid request, successfully transmits the response, reports response success, waits for the normal disconnect interaction, and then performs the centralized cleanup sequence.

Request received
    |
Dispatch successful
    |
Response send successful
    |
"Response packet sent successfully"
    |
Normal disconnect / ENTER
    |
exitCode = 0
    |
Connection::disconnect()
    |
Server::stop()
    |
main() returns 0

## 5.2 Termination after peer RST during workflow

In the GAP-035 scenario, the client resets the connection while the server is paused immediately before response transmission. When the server continues, packetIO.send() reaches the transport send path and returns a send failure. The application detects the error, sets exitCode = 1, breaks out of the main workflow, and still reaches the existing centralized cleanup section.

Request received
    |
Dispatch successful
    |
GDB breakpoint before packetIO.send(response)
    |
Client sends RST
    |
Server continues
    |
packetIO.send(response)
    |
Send failed
    |
"Failed to send response packet: Send failed"
    |
exitCode = 1
    |
Centralized cleanup
    |
Connection::disconnect()
    |
Server::stop()
    |
main() returns 1

The different exit statuses are intentional: the GAP-035 client returns 0 when the test successfully injects and verifies the failure condition, while the server returns 1 because its real request/response workflow experienced a transmission failure.

# 6. Test Approach

The approach was designed to make the failure deterministic and to avoid relying on a race between the client reset and the server send operation.

Freeze the existing production implementation and do not modify the server before observing its current behavior.

Create a GAP-035-specific client that connects to 127.0.0.1:9090, builds a valid HelloRequest, sends it, and then waits at an explicit control point.

Build the server with debug information so that GDB can stop execution at the exact response-send boundary.

Identify the correct application-level send call in apps/server_main.cpp. The targeted request/response send is line 413; line 292 belongs to the protocol-error response path and is not the target for GAP-035.

Run the real packetforge_server under GDB and wait until the request has been received and dispatched.

Allow GDB to stop immediately before packetIO.send(response).

Only after the breakpoint is hit, trigger the GAP-035 client to perform the abortive TCP close (RST).

Resume the server with GDB continue and observe the real packetIO/Transport send path.

Capture the server logs, server exit status, client logs, and client exit status.

Evaluate all acceptance criteria before deciding closure.

# 7. GAP-035-Specific Client

A dedicated executable, apps/packetforge_gap035_client.cpp, was created specifically for this scenario. It does not attempt to receive the server response because the purpose of the test is to terminate the peer before the server can complete that transmission.

| Client step | Behavior | Evidence |
| --- | --- | --- |
| Connection | Connect to 127.0.0.1:9090 | Connection: PASS |
| Request creation | Create valid V1 HelloRequest | HelloRequest validation: PASS |
| Request transmission | Encode and send the request | HelloRequest transmission: PASS |
| Synchronization | Wait until GDB stops the server before response send | Explicit operator control point |
| RST injection | Set SO_LINGER {1,0} and disconnect | TCP reset configuration: PASS; TCP reset injected: PASS |
| Client state | Verify logical state is DISCONNECTED | Connection state after reset: DISCONNECTED |
| Client result | Exit after successful fault injection | Exit status 0 |

Build command used for the dedicated client:

g++ -std=c++20 -Wall -Wextra -Iinclude \
    apps/packetforge_gap035_client.cpp \
    src/common/error.cpp \
    src/common/endian.cpp \
    src/network/socket.cpp \
    src/network/connection.cpp \
    src/network/transport.cpp \
    src/protocol/packet.cpp \
    src/protocol/encoder.cpp \
    -pthread \
    -o bin/packetforge_gap035_client

# 8. GDB Synchronization and Exact Breakpoint

GDB was used to remove the timing race that affected an earlier response-send-failure experiment. The server is paused at the exact application boundary where the response is about to be transmitted.

The source search was performed with:

grep -n -B5 -A8 "packetIO.send(response)" apps/server_main.cpp

The targeted application path was identified as:

408:        std::cout
409:            << "Sending response packet..."
410:            << std::endl;
412:        const auto sendError =
413:            packetIO.send(response);
415:        if (!sendError.ok())

The GDB session was then started with:

gdb ./bin/packetforge_server

(gdb) b 413
(gdb) i b
(gdb) run

The breakpoint was confirmed at apps/server_main.cpp:413. Once the server reached this breakpoint, the server process remained paused while the client injected the RST.

# 9. Step-by-Step Execution Procedure

1. From the PacketForge V1 root directory, verify the targeted source line with grep.

2. Start packetforge_server under GDB.

3. Set breakpoint 413, which is immediately before packetIO.send(response).

4. Run the server and wait for client connection.

5. From a second terminal, launch ./bin/packetforge_gap035_client.

6. Allow the client to connect and send the valid HelloRequest.

7. Verify that GDB stops after the server logs 'Request dispatched successfully' and 'Sending response packet...'.

8. Do not issue GDB continue yet.

9. In the client terminal, press ENTER to execute the SO_LINGER {1,0} abortive close and inject the TCP RST.

10. Return to GDB and execute continue.

11. Observe the real response-send result, cleanup messages, and server exit status.

12. Capture the client exit status with echo $?.

# 10. Captured Execution Evidence

## 10.1 GDB / Server evidence

PacketForge server started successfully
Listening on 127.0.0.1:9090
Waiting for client connection...
Client connected successfully
Connection state: CONNECTED

Waiting for packet...
Packet received successfully
Received request opcode: 1

Dispatching request...
Request dispatched successfully
Sending response packet...

Breakpoint 1, main () at apps/server_main.cpp:413
413                 packetIO.send(response);
(gdb) continue
Continuing.
Failed to send response packet: Send failed
Client connection closed
PacketForge server stopped successfully
[Inferior 1 (process 23194) exited with code 01]

## 10.2 Client evidence

GAP-035 Peer Disconnect During Workflow Client
Target: 127.0.0.1:9090

Connection: PASS
Connection state: CONNECTED
HelloRequest validation: PASS
HelloRequest transmission: PASS

Waiting for GDB breakpoint on the server...
When the server is stopped immediately before
packetIO.send(response), press ENTER here to
inject the peer TCP RST.

TCP reset configuration: PASS
TCP reset injected: PASS
Connection state after reset: DISCONNECTED
GAP-035 peer disconnect injection: PASS

echo $?
0

# 11. Result Analysis

The execution demonstrates that the GAP-035 fault was introduced at the intended point and that the failure propagated through the actual application path.

GAP-035 CLIENT
    |
    | valid request
    v
REAL PACKETFORGE SERVER
    |
    | receive()
    v
Packet received
    |
    | dispatch()
    v
Request dispatched
    |
    | GDB breakpoint
    v
PAUSED BEFORE packetIO.send(response)
    |
    | client RST
    v
packetIO.send(response)
    |
    | Transport::send()
    v
Send failed
    |
    v
server_main detects !sendError.ok()
    |
    v
exitCode = 1
    |
    v
centralized cleanup
    |
    +--> Connection::disconnect()
    |
    +--> Server::stop()
    |
    v
server exits 1

No false 'Response packet sent successfully' message was logged after the send failure. No SIGPIPE termination was observed in this run. The application reached its explicit send-error branch and then executed centralized cleanup.

# 12. Acceptance Criteria

| ID | Acceptance criterion | Result |
| --- | --- | --- |
| 035.1 | Client connects successfully | PASS |
| 035.2 | Valid HelloRequest is transmitted | PASS |
| 035.3 | Server receives the request | PASS |
| 035.4 | Server dispatches the request | PASS |
| 035.5 | Deterministic pause immediately before response send | PASS |
| 035.6 | Client injects TCP RST during the pause | PASS |
| 035.7 | Server resumes into the real response-send path | PASS |
| 035.8 | Send failure is detected and propagated | PASS |
| 035.9 | No false response-success message is emitted | PASS |
| 035.10 | Centralized connection/server cleanup is executed | PASS |
| 035.11 | Server terminates with expected failure status 1 | PASS |
| 035.12 | No hang or uncontrolled termination occurs | PASS |

# 13. Normal vs RST Termination Summary

| Aspect | Normal successful workflow | GAP-035 RST workflow |
| --- | --- | --- |
| Peer behavior | Orderly close / normal completion | Abortive TCP reset |
| Response send | Succeeds | Fails |
| Application log | Response packet sent successfully | Failed to send response packet: Send failed |
| Application exitCode | 0 | 1 |
| Cleanup | Connection::disconnect() then Server::stop() | Same centralized cleanup |
| Server process | Exits normally after successful workflow | Exits with expected failure status after handling send failure |
| Test-client status | Not applicable | 0 means fault injection/validation succeeded |

This distinction is important: server exit status 1 in GAP-035 is not a test failure. It is the expected process status for the server because the workflow failed to transmit its response. The dedicated test client returns 0 because it successfully created the fault condition required by the test.

# 14. Production Code Impact

No production architecture or behavior change was required as a result of this validation.

No changes to the synchronous/blocking V1.0 networking architecture.

No select(), poll(), epoll(), event loop, or non-blocking socket introduced.

No retry or artificial delay added to the server.

No special production test hook added to server_main.cpp.

Existing packetIO.send() error handling and centralized cleanup were exercised as-is.

The GAP-035-specific client exists only as a validation executable.

# 15. Closure Decision

GAP-035 is CLOSED / PASS.

The test established a deterministic peer disconnect during the active request/response workflow. The real PacketForge server was paused immediately before response transmission, the client injected a TCP RST, and the server subsequently detected the send failure through the existing PacketIO/Transport path. The server did not report false transmission success, performed centralized cleanup, and terminated with exit status 1 as defined for the failed workflow. The client independently verified successful RST injection and returned exit status 0.

No production defect requiring a GAP-035 code change was exposed by this execution.

# 16. Reproducibility Record

| Item | Recorded value |
| --- | --- |
| Server executable | bin/packetforge_server |
| GAP-specific client | bin/packetforge_gap035_client |
| Endpoint | 127.0.0.1:9090 |
| GDB version | GNU gdb 15.0.50.20240403 |
| Breakpoint | apps/server_main.cpp:413 |
| Target call | packetIO.send(response) |
| Fault mechanism | SO_LINGER {1,0} followed by Connection::disconnect() |
| Server result | Send failed; exit code 1 |
| Client result | RST injected successfully; exit code 0 |